# S- CAM (CAMSHAFTS)

## 1-1/2” – 10 Spline, Standard “S” Head Camshafts

| Left Hand As viewed from Head |  | Dimension A is from shoulder of camshaft to far edge of snap ring groove |  |  |  | Right Hand As viewed from Head |
| --- | --- | --- | --- | --- | --- | --- |
| PART NO. |  |  | DIMENSIONS |  | APPLICATION AND GENERAL INFORMATION |  |
| LEFT HAND | RIGHT HAND |  | A | B |  |  |
| 03.0112.L | 03.0112.R |  | 8.69" = 220.7mm | 1.97" = 50.0mm | For Eaton and Mack Drive Axles Replaces: Eaton 1000087 (LH) & 1000088 (RH) |  |
| 03.0121.L | 03.0121.R |  | 11.66" = 296.2mm | 1.69" = 42.9mm | For Eaton Drive Axles Replaces: Eaton 70306 (LH) & 70307 (RH) |  |
| 03.0125.L | 03.0125.R |  | 12.56" = 319.0mm | 1.56" = 39.6mm | For Eaton Drive Axles Replaces: Eaton 71306 (LH) & 71307 (RH) |  |
| 03.0131.L | 03.0131.R |  | 13.72" = 348.5mm | 3.50" = 88.9mm | For Eaton Drive Axles Replaces: Eaton 35455 (LH) & 35456 (RH) IHC 109112R1 (LH) & IHC 109113R1 |  |

Australian Transport Manufacturers Pty Ltd manufacture standard ‘S’ head camshafts, 10 or 28 spline, to order.  Maximum Length (excluding ‘S’ head): 20.5” = 520.7mm

Camshafts can be manufactured from a sample or from a drawing.

All parts are manufactured to Australian standards.

## 1-1/2” – 10 Spline, Standard “S” Head Camshafts

| Left Hand As viewed from Head |  | Dimension A is from shoulder of camshaft to far edge of snap ring groove |  |  |  | Right Hand As viewed from Head |
| --- | --- | --- | --- | --- | --- | --- |
| PART NO. |  |  | DIMENSIONS |  | APPLICATION AND GENERAL INFORMATION |  |
| LEFT HAND | RIGHT HAND |  | A | B |  |  |
| 03.0181.L | 03.0181.R |  | 4.09" = 103.9mm | full bearing |  |  |
| 03.0202.L | 03.0202.R |  | 6.52" = 165.6mm | 1.75" = 44.5mm | For Rockwell Drive Axles Replaces: Rockwell 2210-X-3066 (LH) & 2210-Y-3067 (RH) Rockwell 2210-B-5722 (LH) & 2210-A-5721 (RH) |  |
| 03.0203.L | 03.0203.R |  | 7.30" = 185.4mm | full bearing |  |  |
| 03.0204.L | 03.0204.R |  | 8.03" = 204.0mm | 1.75" = 44.5mm | For Rockwell Drive Axles Replaces: Rockwell 221 0-J-3026 (LH) & 221 0-H-3024 (RH) Rockwell 2210-X-5718 (LH) & 2210-W-5717 (RH) |  |
| 03.0206.L | 03.0206.R |  | 8.19" = 208.0mm | 1.75" = 44.5mm | For Rockwell Drive Axles Replaces: Rockwell 2210-F-5544 (LH), & 2210-E-5543 (RH) For 03.0206.L with C&W Kit 01.0123.00 Order 03.0206.LK For 03.0206.R with C&W Kit 01.0123.00 Order 03.0206.RK |  |
| 03.0210.L | 03.0210.R |  | 8.65" = 219.7mm | 1.75" = 44.5mm | For Mack Trucks using 38,000 lb. and 44,000 lb. Rockwell Axles For Rockwell Drive Axles Replaces: Rockwell 2210-Y-2443 (LH) & 2210-X-2442 (RH) Rockwell 2210-X-5640 (LH) & 2210-W-5639 (RH) |  |
| 03.0213.L | 03.0213.R |  | 9.19" = 233.4mm | 1.91" = 48.5mm | For Rockwell Drive Axles Replaces: Rockwell 2210-A-4265 (LH) & 2210-2-4264 (RH) 2210-H-5546 (LH) & 2210-G-5545 (RH) |  |
| 03.0214.L | 03.0214.R |  | 9.37" = 238.0mm | 2.25" = 57.2mm | For Rockwell Drive Axles Replaces: Rockwell 2210-P-2434 (LH) & 2210-0-2435 (RH) Rockwell 2210-B-5384 (LH) & 2210-A-5383 (RH) |  |
| 03.0216.L | 03.0216.R |  | 9.85" = 250.2mm | 1.87" = 47.5mm | For Denning Bus Rockwell 2210-W-3741 (LH) & 2210-X-3742 (RH) |  |
| 03.0218.L | 03.0218.R |  | 10.31" = 261.9mm | 1.87" = 47.5mm | For Eaton Drive Axles and Eaton Self Contained Trailer Axles For Rockwell Drive Axles Replaces: Eaton B-1735-1 (LH) & B-1735-2 (RH) Rockwell 2210-V-2440 (LH) & 2210-W-2441 (RH) Rockwell 2210-E-5699 (LH) & 2210-F-5700 (RH) |  |
| 03.0222.L | 03.0222.R |  | 11.33" 287.8mm | 1.90" = 48.3mm | For Rockwell Drive Axles Replaces: Rockwell 2210-L-3028 (LH) & 2210-K-3027 (RH) Rockwell 2210-X-5354 (LH) & 2210-W-5353 (RH) |  |
| 03.0226.L | 03.0226.R |  | 11.62" = 295.1mm | 1.87" = 47.5mm | For Mack Drive Axles For Rockwell Drive Axles For Standard Forge Drive Axles Replaces: Rockwell 2210-P-2980 (LH) & 2210-0-2981 (RH) Rockwell 2210-D-5360 (LH) & 221 0-C-5359 (RH) Std. Forge 202199-X9-116L (LH) & 202199-X9-116R (RH) |  |
| 03.0230.L | 03.0230.R |  | 12.07" = 306.6mm | 1.81" = 46.0mm | For Rockwell Drive Axles For Wagner 15" Dia. Drive Axle Air Brakes Replaces: Wagner FE31945 (LH) & FE31944 (RH) Rockwell 2210-R-5348 (LH) & 2210-0-5347 (RH |  |
| 03.0234.L | 03.0234.R |  | 12.56" = 318.9mm | 1.75" = 44.5mm | For Rockwell Drive Axles Replaces: Rockwell 221 0-L-2404 (LH) & 221 0-M-2405 (RH) |  |
| 03.0236.L | 03.0236.R |  | 13.77" = 349.8mm | 1.85" = 47.0mm | For Rockwell Drive Axles Replaces: Rockwell 2210-X-5380 (LH) & 2210-W-5379 (RH) |  |
| 03.0237.L | 03.0237.R |  | 14.18"= 360.2mm | 1.85" = 47.0mm | For Rockwell Drive Axles Replaces: Rockwell 2210-H-5390 (LH) & 2210-G-5389 (RH) |  |
| 03.0238.L | 03.0238.R |  | 15.75" = 400.1mm | 1.87" = 47.5mm | For Rockwell Drive Axles Replaces: Rockwell 2210-N-2406 (LH) & 2210-P-2408 (RH) Rockwell 221 0-V-5638 (LH) & 221 0-U-5637 (RH) |  |

## 1-1/2” – 10 Spline, Standard “S” Head Camshafts

| Left Hand As viewed from Head |  | Dimensions A&B are from shoulder of camshaft to far edge of snap ring groove |  |  |  | Right Hand As viewed from Head |
| --- | --- | --- | --- | --- | --- | --- |
| PART NO. |  |  | DIMENSIONS |  | APPLICATION AND GENERAL INFORMATION |  |
| LEFT HAND | RIGHT HAND |  | A | B |  |  |
| 03.0240.L | 03.0240.R |  | 16.68" = 423.7mm | 2.27" = 57.7mm | For "Trade" Trailer Axle 8rakes Replaces: Eaton 8-1691-47 (LH) & 8-1691-48 (RH) Rockwell 2210-S-2489 (LH) & 2210-R-2488 (RH) Std. Forge 202100-166L (LH) & 202100-166R (RH) |  |
| 03.0260.L | 03.0260.R |  | 20.50" = 520.7mm | 2.27" = 57.7mm | For ''Trade'' Trailer Axle 8rakes Replaces: Dana 11239 (LH) & 11238 (RH) Dana 16392 (LH) & 16391 (RH) Eaton 8-1283-5 (LH) & 8-1283-6 (RH) Eaton 8-1817-1 (LH) & 8-1817-2 (RH) Freighter 154-00769 (LH) & 154-00768 (RH) Fruehauf 219316 (LH) & 219317 (RH) Fruehauf 580443 (LH) & 580444 (RH) Rockwell 2210-N-2484 (LH) & 2210-M-2483 (RH) Rockwell 2210-G-3595 (LH) & 2210-F-3594 (RH) Rockwell 2210-F-5414 (LH) & 2210-E-5413 (RH) R.O.R. 2120-5407 (LH) & 2120-5406 (RH) Std. Forge 202100-205L (LH) & 202100-205R (RH) |  |
| 03.0274.L | 03.0274.R |  | 24.44" = 620.8mm | 2.27" = 57.7mm | For "Trade" Trailer Axle 8rakes Replaces: Dana 16844 (LH) & 16843 (RH) Eaton 8-1691-51 (LH) & 8-1691-52 (RH) Rockwell 2210-U-3375 (LH) & 2210-T-3374 (RH) |  |

## 1-1/2” – 10 Spline, Small “S” Head Camshafts

| Left Hand As viewed from Head |  | Dimensions A,B&C are from shoulder of camshaft to far edge of snap ring groove |  |  |  |  | Right Hand As viewed from Head |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PART NO. |  |  | DIMENSIONS |  |  | APPLICATION AND GENERAL INFORMATION |  |
| LEFT HAND | RIGHT HAND |  | A | B | C |  |  |
| 03.0318.L | 03.0318.R |  | 21.19" = 538.2mm | 2.28" = 57.9mm | 2.72" = 69.1mm | For Dana Trailer Axle with 12-1/4" Dia. Brakes For Std. Forge Trailer Axle with 12-1/4" Dia. Brakes Replaces: Dana 11594 (LH) & 11593 (RH) Dana 12206 (LH) & 12205 (RH) Std. Forge 2021 03-213L (LH) & 2021 03-213R (RH) Std. Forge UB22-64L (LH) & UB22-64R (RH) |  |
| 03.0336.L | 03.0336.R |  | 21.19" = 538.2mm | 2.28" = 57.9mm | 2.72" = 69.1mm | Std. Forge 2021 03-233L (LH) & 2021 03-233R (RH) |  |

## 1-1/2” – 28 Spline, Standard “S” Head Camshafts

| Left Hand As viewed from Head |  | Dimension A is from shoulder of camhead to far edge of snap ring groove |  |  |  | Right Hand As viewed from Head |
| --- | --- | --- | --- | --- | --- | --- |
| PART NO. |  |  | DIMENSIONS |  | APPLICATION AND GENERAL INFORMATION |  |
| LEFT HAND | RIGHT HAND |  | A | B |  |  |
| 03.0510.L | 03.0510.R |  | 8.63" = 219.2mm | 1.79" = 45.5mm | For Mack Truck Axles For Rockwell Drive Axles Replaces: Rockwell 2210-F-5440 (LH) & 2210-E-5439 (RH) |  |
| 03.0514.L | 03.0514.R |  | 9.16" = 232.7mm | 1.79" = 45.5mm | For Mack Truck Axles For Rockwell Drive Axles Replaces: Rockwell 2210-V-5456 (LH) & 2210-U-5455 (RH) |  |
| 03.0516.L | 03.0516.R |  | 9.35" = 237.5mm | 1.79" = 45.5mm | For Rockwell Drive Axles Replaces: Rockwell 2210-R-5374 (LH) & 2210-0-5373 (RH) |  |
| 03.0522.L | 03.0522.R |  | 11.30" = 287.0mm | 1.87" = 47.5mm | For Rockwell Drive Axles Replaces: Rockwell 2210-D-6036 (LH) & 2210-C-6035 (RH) |  |
| 03.0526.L | 03.0526.R |  | 11.60" = 294.6mm | 1.87" = 47.5mm | For Rockwell Drive Axles Replaces: Rockwell 2210-B-5332 (LH) & 2210-A-5331 (RH) |  |
| 03.0528.L | 03.0528.R |  | 11.72" = 297.7mm | 1.79" = 45.5mm | For Mack Truck Axles For Rockwell Drive Axles Replaces: Rockwell 2210-H-5416 (LH) & 2210-G-5415 (RH) |  |
| 03.0532.L | 03.0532.R |  | 12.05" = 306.1mm | 1.79" = 45.5mm | For Rockwell Drive Axles Replaces: Rockwell 221 0-L-7552 (LH) & 221 0-K-7551 (RH) |  |
| 03.0536.L | 03.0536.R |  | 13.75" = 349.3mm | 1.79" = 45.5mm | For Rockwell Drive Axles Replaces: Rockwell 2210-T-5454 (LH) & 2210-8-5453 (RH) |  |
| 03.0537.L | 03.0537.R |  | 14.16" = 359.7mm | 1.79" = 45.5mm | For Rockwell Drive Axles Replaces: Rockwell 2210-N-5370 (LH) & 2210-M-5369 (RH) |  |
| 03.0539.L | 03.0539.R |  | 15.30" = 388.6mm | 1.89" = 48.0mm | For Rockwell Drive Axles Replaces: Rockwell 2210-A-7671 (LH) & 2210-Z-7670 (RH) |  |
| 03.0540.L | 03.0540.R |  | 15.44" = 422.3mm | 1.89" = 48.0mm | Camshaft L/H 28 Spline 15.44" Long |  |
| 03.0545.L | 03.0545.R |  | 16.63" = 422.3mm | 1.89" = 48.0mm | Camshaft R/H 28 Spline 15.44" Long |  |

## 1-1/2” – 10 Spline, Standard “S” Head Camshafts

## Heavy Duty Cams  for use with Tayco Lock-On Slack Adjusters

| Left Hand As viewed from Head |  | Dimension C is from shoulder of camhead to far edge of snap ring groove *Part Numbers include the Lock Nut |  |  |  |  |  | Right Hand As viewed from Head |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PART NO. |  |  | DIMENSIONS |  |  |  | APPLICATION AND GENERAL INFORMATION |  |
| LEFT HAND | RIGHT HAND |  | A | B | C | D |  |  |
| 03.1210.L | 03.1210.R |  | 205.1mm | 45mm | 162.6mm | 22mm | For Mack Heavy Duty Version of 03.021 O.L & 03.021 O.R |  |